TfL inspection and maintenance intervention levels

The TfL road network comprising, footways, carriageways & cycleways are inspected at regular intervals by a team of trained highway inspectors at frequencies varying between weekly and monthly depending on usage.  The inspectors record defects and prioritisation according to severity follows.

Defects posing cycle specific hazards are identified as follows within TfL highway features:

Carriageway

- A pothole 20mm or deeper over 100sqcm or more within 1.5m of the kerb or within a formally marked cycle lane

- A pothole 30mm or deeper over 100sqcm or more elsewhere

- Spalling of concrete 20mm or deeper over 400sqcm or more

- Crowning of 40mm or more over a 3m length

- A depression of 40mm or more within a 1m length or 25mm or more within a 300mm length

- Rutting of 40mm or more

- A gap or crack 20mm or wider, 40mm or deeper and 500mm or longer

- An oil or diesel spill over 1sqm

- Missing or defective anti-skid surfacing over 1sqm

- Standing water 10mm or deeper over 500mm in width adjacent to the kerb or 20mm or deeper over 1sqm or more elsewhere

- Debris, building materials, abandoned vehicles or other obstruction likely to create a hazard

- Inadequate signing or guarding of works

Pedestrian Crossing

- A ‘trip’ of 20mm or more

Footway/Shared Path/Cycle Track

- A ‘trip’ of 20mm or more

- A pothole 20mm or deeper over 100sqcm or more

- A rocking slab or block with 20mm or more movement

- A gap or crack 20mm or wider, 20mm or deeper and 200mm or longer

- Standing water 10mm or deeper over 1sqm or more

- Cellar or other access doors or vents likely to create a hazard

- Damaged, misaligned or defective street furniture likely to create a hazard

- Height clearance less than 2.5m to cycle path or cycle track below signs or overhanging trees or vegetation

- Height clearance less than 2.1m to footway below signs or overhanging trees or vegetation

- A tree base 20mm or more below footway level

- A damaged or defective tree grid likely to create a hazard

- Advertising, scaffolding, hoarding, building materials, vegetation or other obstruction likely to create a hazard

- Inadequate signing or guarding of works

Cycle route / shared path

All as other features with the addition of:-

- A longitudinal gap or crack 10mm or wider, 40mm or deeper and 500mm or longer

- An oil or diesel spill over 300mm diameter

- A ‘trip’ of 20mm or more

- Gully grating unsuitable for cycling

- Minimum headroom clearance for cyclists less than 2.5m

Kerbing

- A unit dislodged by 50mm or more horizontally

- A unit sunk by 20mm or more compared to an adjacent unit

- A unit rocking with 20mm or more of movement

- A missing unit

Ironwork

- A broken or cracked cover likely to create a hazard

- A worn or polished cover likely to create a hazard

- A missing cover

- A rocking cover or frame likely to cause a hazard or noise nuisance

- Ironwork sunk or projecting by 20mm or more

- Fluid discharging and likely to create a health or safety hazard

- A missing gully grate

- A blocked gully likely to create a hazard

- A broken or cracked gully grate likely to create a hazard

Defect Response

Once a defect has been identified, it is assessed to determine the likelihood of injury. This will depend on its severity, location and likely level of usage by road users. The most urgent defects are made safe within an hour. Other defects are usually required to be repaired within 24 hours, 7 days or 28 days depending on an assessment of their risk.